## Appendix 1 – Risk Assessment, Description, Evaluation And Control (RADEC) – EXAMPLE

| RISK ASSESSMENT, DESCRIPTION, EVALUATION AND CONTROL (RADEC) FORM |  |  |
| --- | --- | --- |
| RA No.: H001 | Definition: Confined area landing |  |
| Ref.: Sling load Standard Operating Procedures |  |  |
| Operation Description: Sling load operations in mountainous areas |  |  |
| Hazards - What were or could be the sources of potential damage, harm or adverse health effects in the studied environment? - Trees and vegetation - Wires, power lines - Meteorological conditions - Wind, turbulence, downdrafts - Confined landing sites |  |  |
| Possible Hazard Consequences - What were or could be the hazard consequences? - Main rotor strikes ground obstacles - Tail rotor strikes ground obstacles - In flight contact with wires, power lines - Ground personnel injuries due to lifted loads or helicopter downdraft - Inadvertent or accidental cargo release - Damage on ground - Power loss in flight - Power settling - Uncontrolled cargo swing |  |  |
| Controls in place - What risk controls are already in place to address these? - Minimum size of the landing area (25 m x 25 m) - Pilot experience: minimum 500 flight hours in aerial work. - Area must be inspected by Company personnel before landing - High and low recognition before the first landing - Sling load operating procedures - Wire, power lines area mapped before sling load operations |  |  |
| INITIAL Safety Risk - Refer to the Safety Risk Matrix (if you use one) |  |  |
| ACCEPTABLE | TOLERABLE | UNACCEPTABLE |

| Additional Controls - What more should be done to further reduce the initial safety risks to an acceptable level? - Personnel on ground to be in radio contact with the pilot - Personnel on ground to wear helmets and personal safety equipment - Minimum cable length = length of higher obstacle + 15 feet - Maximum take-off weight with cargo reduced by 5% |  |  | Implemented? YES YES YES NO (on-going) |
| --- | --- | --- | --- |
| FINAL Safety Risk (see Safety Risk Matrix) |  |  |  |
| ACCEPTABLE | TOLERABLE | UNACCEPTABLE |  |
| Is the residual risk acceptable: YES NO (if NO go back to previous section) RISK ASSESSMENT CLOSED |  |  |  |