Courtesy: Elicampiglio

Photo: Michele Calovi

EHEST SAFETY MANAGEMENT TOOLKIT

HAZARDOUS EVENT IDENTIFICATION & RISK ASSESSMENT

HIRA - EXAMPLE – TABULAR

1st Edition, December 2015

HAZARDOUS EVENT IDENTIFICATION & RISK ASSESSMENT

HIRA 001

Sling Load Operations

(Change numbering and title accordingly)

- - E X A M P L E - -

Edition 1

December 2015

| Signature |  |  |  |  |
| --- | --- | --- | --- | --- |
| Position |  |  |  |  |
| Name |  |  |  |  |
|  | Issued by | Checked by | Approved by | Approved by |

NOTE

The following example and its related tables come from the “Report Op-Proact. Barr.” output of the EHEST MARIA database tool, Sling Load Operations example.

Due to multiple connections between the elements, the table becomes significally long.

The operator can use the MARIA tool and create the table from the related report or he can define his own table elements manually, sometimes drastically reducing the table complexity.

The key points of the document are to:

- define the HAZARDOUS EVENTS related to the operation;

- define the possible CONSEQUENCES;

- define the BARRIERS in order to reduce the RISK;

- RATE the LIKELIHOOD of the Hazardous Events and the RISK of the Consequences; and

- apply the barriers to the SOP.

The table “2. HIRA – CAUSE-HAZARDOUS EVENT” is not strictly mandatory, but it helps in defining the PROACTIVE BARRIERS.

For regulatory purposes, only table “3. HIRA – HAZARDOUS EVENT-CONSEQUENCE” is required, provided that both PROACTIVE and RECOVERY barriers are reported.

The tables are compatible with GM3 ORO.GEN.200(a)(3) Management system and AMC2 SPO.OP.230 Standard operating procedures.

# INDEX

1.	INDEX	4

2.	HIRA – CAUSE-HAZARDOUS EVENT	5

3.	HIRA – HAZARDOUS EVENT-CONSEQUENCE	38

4.	LIST OF BARRIERS	54

# HIRA – CAUSE-HAZARDOUS EVENT

OPERATION No.: OP 01	OPERATION DESCRIPTION: Sling load operations

| HE REF. | CAUSE REF. | PROACT. BAR. REF. | DESCRIPTIONS | LIKELI-HOOD |
| --- | --- | --- | --- | --- |
| HE 001 |  |  | Reduced separation with obstacles | 2 |
|  | CA 001 |  | Trees, bushes, vegetation |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 002 |  | Wires, power lines |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 003 |  | Low Clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 004 |  | Low visibility, fog |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 002 |  |  | Reduced separation of the load from obstacles (sling load) | 3 |
|  | CA 001 |  | Trees, bushes, vegetation |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 002 |  | Wires, power lines |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 008 |  | Properties on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 003 |  |  | Degraded aircraft performance | 1 |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 014 |  | Maintenance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 004 |  |  | Degraded aircraft performance during sling load operations | 1 |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 014 |  | Maintenance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 005 |  |  | Aircraft unintentionally deviates from normal in-flight parameters | 2 |
|  | CA 003 |  | Low Clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 004 |  | Low visibility, fog |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 005 |  | Vertical development clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 006 |  | Wind |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
| HE 006 |  |  | Safety measures not respected by personnel on ground during sling load operations | 3 |
|  | CA 007 |  | Personnel on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 011 |  | Lose objects on the ground |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 012 |  | Sand, dust, light soil |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 016 |  | Personnel proficiency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 018 |  | Radio communications |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  | CA 020 |  | Customer |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
| HE 007 |  |  | Load is not properly prepared (sling load operations) | 3 |
|  | CA 007 |  | Personnel on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 008 |  | Properties on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 016 |  | Personnel proficiency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 008 |  |  | Unexpected technical problem related to sling load operations | 3 |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 014 |  | Maintenance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 016 |  | Personnel proficiency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 009 |  |  | Unexpected technical problem on aircraft | 3 |
|  | CA 014 |  | Maintenance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 010 |  |  | Lose objects moved by the rotor downwash | 3 |
|  | CA 001 |  | Trees, bushes, vegetation |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 007 |  | Personnel on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 008 |  | Properties on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 011 |  | Lose objects on the ground |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 012 |  | Sand, dust, light soil |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 016 |  | Personnel proficiency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
| HE 011 |  |  | Not accurate or missing mission planning and preparation (sling load) | 2 |
|  | CA 001 |  | Trees, bushes, vegetation |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 002 |  | Wires, power lines |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 003 |  | Low Clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 004 |  | Low visibility, fog |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 005 |  | Vertical development clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 006 |  | Wind |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  | CA 007 |  | Personnel on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 008 |  | Properties on ground |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 009 |  | Confined area operations, landings, take offs |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 011 |  | Lose objects on the ground |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 016 |  | Personnel proficiency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 020 |  | Customer |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
| HE 012 |  |  | Flight close to, or into, adverse meteorological conditions | 2 |
|  | CA 003 |  | Low Clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 004 |  | Low visibility, fog |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 005 |  | Vertical development clouds |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 005 | IFR training |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  | CA 006 |  | Wind |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 019 |  | Mountainous area |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
| HE 013 |  |  | Unwanted movement of cargo in flight (sling load) | 2 |
|  | CA 006 |  | Wind |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 006 | Meteorological information |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  | CA 010 |  | Use of sling load |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 025 | EGPWS |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 013 |  | External load size and weight |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 002 | Sling load operation training |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 026 | Use of mirrors (sling load) |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
| HE 014 |  |  | Repetitive flight patterns/manoeuvres | 4 |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 020 |  | Customer |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
| HE 015 |  |  | External pressure | 3 |
|  | CA 015 |  | Pilot proficiency |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 004 | Pilot recency |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 011 | Pilot minimum mountain experience |  |
|  |  | PB 012 | Emergency procedures training |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 021 | Area qualification |  |
|  |  | PB 023 | IFR rated pilot |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 016 |  | Personnel proficiency |  |
|  |  | PB 007 | Personal Protective Equipment (PPE) |  |
|  |  | PB 008 | Radio contact with personnel on ground |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 015 | Personnel on ground recency |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 017 |  | Operation planning |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 003 | Pilot experience |  |
|  |  | PB 009 | Initial Recce of the operating area |  |
|  |  | PB 010 | Previous ground reconnaissance |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 014 | Personnel on ground experience |  |
|  |  | PB 016 | Mission planning |  |
|  |  | PB 017 | Adverse weather policy/SOP |  |
|  |  | PB 018 | Weather reports |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 027 | Use of suitable equipment |  |
|  |  | PB 028 | Service and duty time |  |
|  | CA 020 |  | Customer |  |
|  |  | PB 001 | SOP Sling Load Operations |  |
|  |  | PB 013 | Maintenance planning |  |
|  |  | PB 019 | Pre-flight Risk Assessment |  |
|  |  | PB 020 | In-flight decision making training |  |
|  |  | PB 022 | Continuous Airworthiness Organisation (CAMO) |  |
|  |  | PB 024 | Safety Reports |  |
|  |  | PB 028 | Service and duty time |  |

# HIRA – HAZARDOUS EVENT-CONSEQUENCE

OPERATION No.: OP 01	OPERATION DESCRIPTION: Sling load operations

| HE REF. | CONSEQ. REF. | REC. BAR. REF. | DESCRIPTIONS | LIKELIHOOD | RISK |
| --- | --- | --- | --- | --- | --- |
| HE 001 |  |  | Reduced separation with obstacles | 2 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 004 |  | Injury to flight crew or crew members |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 006 |  | Aircraft not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 007 |  | Pilot/personnel not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  | CO 009 |  | Rules infringement |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 011 |  | Main rotor blades strike a ground obstacle |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 012 |  | Tail rotor blades strike a ground obstacle |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 013 |  | Impact with wires or cables |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
| HE 002 |  |  | Reduced separation of the load from obstacles (sling load) | 3 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 004 |  | Injury to flight crew or crew members |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 006 |  | Aircraft not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 007 |  | Pilot/personnel not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  | CO 009 |  | Rules infringement |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 011 |  | Main rotor blades strike a ground obstacle |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 012 |  | Tail rotor blades strike a ground obstacle |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 013 |  | Impact with wires or cables |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 017 |  | Payload damage during flight |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 018 |  | Sling cable or load contact with obstacle on ground |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 019 |  | Sling cable or load get caught with obstacle on ground |  | 2 C |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 003 |  |  | Degraded aircraft performance | 1 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 004 |  | Injury to flight crew or crew members |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 010 |  | Personnel grounded in an impervious area |  | 2 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 011 |  | Main rotor blades strike a ground obstacle |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 015 |  | Loss of tail rotor effectiveness |  | 1 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 028 |  | Unexpected reduced performance |  | 2 C |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 004 |  |  | Degraded aircraft performance during sling load operations | 1 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 004 |  | Injury to flight crew or crew members |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 010 |  | Personnel grounded in an impervious area |  | 2 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 011 |  | Main rotor blades strike a ground obstacle |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 015 |  | Loss of tail rotor effectiveness |  | 1 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 017 |  | Payload damage during flight |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 018 |  | Sling cable or load contact with obstacle on ground |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 019 |  | Sling cable or load get caught with obstacle on ground |  | 2 C |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 022 |  | Emergency cargo release |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 028 |  | Unexpected reduced performance |  | 2 C |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 005 |  |  | Aircraft unintentionally deviates from normal in-flight parameters | 2 |  |
|  | CO 004 |  | Injury to flight crew or crew members |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 009 |  | Rules infringement |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 014 |  | CFIT |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 016 |  | Loss of control with subsequent ground impact or aircraft damage |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  |  | RB 015 | Unusual attitude training |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 030 |  | Sling cable or load contact with the aircraft structure during flight |  | 1 D |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 006 |  |  | Safety measures not respected by personnel on ground during sling load operations | 3 |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 003 |  | Injury to personnel on ground |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 007 |  | Pilot/personnel not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  | CO 009 |  | Rules infringement |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 010 |  | Personnel grounded in an impervious area |  | 2 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 019 |  | Sling cable or load get caught with obstacle on ground |  | 2 C |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 023 |  | Unintentional cargo release |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 024 |  | Object detached lose from cargo |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  | CO 025 |  | FOD |  | 2 B |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 028 |  | Unexpected reduced performance |  | 2 C |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 029 |  | Load swings in flight |  | 3 B |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 030 |  | Sling cable or load contact with the aircraft structure during flight |  | 1 D |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 007 |  |  | Load is not properly prepared (sling load operations) | 3 |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 003 |  | Injury to personnel on ground |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 016 |  | Loss of control with subsequent ground impact or aircraft damage |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  |  | RB 015 | Unusual attitude training |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 017 |  | Payload damage during flight |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 019 |  | Sling cable or load get caught with obstacle on ground |  | 2 C |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 022 |  | Emergency cargo release |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 023 |  | Unintentional cargo release |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 024 |  | Object detached lose from cargo |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  | CO 029 |  | Load swings in flight |  | 3 B |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 030 |  | Sling cable or load contact with the aircraft structure during flight |  | 1 D |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 008 |  |  | Unexpected technical problem related to sling load operations | 3 |  |
|  | CO 006 |  | Aircraft not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 010 |  | Personnel grounded in an impervious area |  | 2 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 017 |  | Payload damage during flight |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 018 |  | Sling cable or load contact with obstacle on ground |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 019 |  | Sling cable or load get caught with obstacle on ground |  | 2 C |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 022 |  | Emergency cargo release |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 023 |  | Unintentional cargo release |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 024 |  | Object detached lose from cargo |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 029 |  | Load swings in flight |  | 3 B |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 030 |  | Sling cable or load contact with the aircraft structure during flight |  | 1 D |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 009 |  |  | Unexpected technical problem on aircraft | 3 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 003 |  | Injury to personnel on ground |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 004 |  | Injury to flight crew or crew members |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 006 |  | Aircraft not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 010 |  | Personnel grounded in an impervious area |  | 2 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 016 |  | Loss of control with subsequent ground impact or aircraft damage |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  |  | RB 015 | Unusual attitude training |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 021 |  | Engine failure – Single-engine |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 010 |  |  | Lose objects moved by the rotor downwash | 3 |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 003 |  | Injury to personnel on ground |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 021 |  | Engine failure – Single-engine |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 025 |  | FOD |  | 2 B |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 028 |  | Unexpected reduced performance |  | 2 C |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 011 |  |  | Not accurate or missing mission planning and preparation (sling load) | 2 |  |
|  | CO 002 |  | Property damage on ground |  | 2 C |
|  |  | RB 010 | Insurance |  |  |
|  | CO 003 |  | Injury to personnel on ground |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 009 |  | Rules infringement |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 013 |  | Impact with wires or cables |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 017 |  | Payload damage during flight |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 022 |  | Emergency cargo release |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 023 |  | Unintentional cargo release |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 028 |  | Unexpected reduced performance |  | 2 C |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 012 |  |  | Flight close to, or into, adverse meteorological conditions | 2 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 005 |  | Injury to passengers |  | 2 C |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 006 |  | Aircraft not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 007 |  | Pilot/personnel not available |  | 3 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  | CO 009 |  | Rules infringement |  | 3 B |
|  |  | RB 004 | Safety reports |  |  |
|  | CO 010 |  | Personnel grounded in an impervious area |  | 2 B |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 013 |  | Impact with wires or cables |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |
|  | CO 014 |  | CFIT |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 016 |  | Loss of control with subsequent ground impact or aircraft damage |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  |  | RB 015 | Unusual attitude training |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 028 |  | Unexpected reduced performance |  | 2 C |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 013 |  |  | Unwanted movement of cargo in flight (sling load) | 2 |  |
|  | CO 016 |  | Loss of control with subsequent ground impact or aircraft damage |  | 1 D |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 003 | Safety Management System (SMS) |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  |  | RB 015 | Unusual attitude training |  |  |
|  |  | RB 016 | IFR training |  |  |
|  |  | RB 017 | Inadvertent IMC training |  |  |
|  | CO 017 |  | Payload damage during flight |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 022 |  | Emergency cargo release |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 023 |  | Unintentional cargo release |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
|  | CO 024 |  | Object detached lose from cargo |  | 2 B |
|  |  | RB 010 | Insurance |  |  |
|  | CO 026 |  | Unplanned landing |  | 2 B |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  | CO 029 |  | Load swings in flight |  | 3 B |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
|  | CO 030 |  | Sling cable or load contact with the aircraft structure during flight |  | 1 D |
|  |  | RB 014 | Recurrent emergency procedures training |  |  |
| HE 014 |  |  | Repetitive flight patterns/manoeuvres | 4 |  |
|  | CO 023 |  | Unintentional cargo release |  | 2 B |
|  |  | RB 004 | Safety reports |  |  |
|  |  | RB 010 | Insurance |  |  |
| HE 015 |  |  | External pressure | 3 |  |
|  | CO 001 |  | Aircraft crash |  | 1 E |
|  |  | RB 001 | Emergency Response Plan (ERP) |  |  |
|  |  | RB 002 | Pilot and personnel data |  |  |
|  |  | RB 005 | Spare helicopter availability |  |  |
|  |  | RB 006 | Personnel proper clothing |  |  |
|  |  | RB 007 | Survival kit |  |  |
|  |  | RB 008 | Portable aeronautical radio on board |  |  |
|  |  | RB 009 | Emergency Locator Beacon (ELT) |  |  |
|  |  | RB 010 | Insurance |  |  |
|  |  | RB 011 | First Aid Kit |  |  |
|  |  | RB 018 | Wire strike protection system |  |  |

# LIST OF BARRIERS

OPERATION No.: OP 01	OPERATION DESCRIPTION: Sling load operations

| BAR. REF. | DESCRIPTION | REF. IN SOP/DOCUMENTS |
| --- | --- | --- |
| PB 001 | SOP Sling Load Operations | SOP 001 - Sling Load Operations |
| PB 002 | Sling load operation training | SOP 001 - Sling Load Operations - 3. (e) (2) - 3. (e) (4) - 3. (f) (1) - 3. (f) (2) OM D - (...) |
| PB 003 | Pilot experience | SOP 001 - Sling Load Operations - 3. (e) (3) |
| PB 004 | Pilot recency | SOP 001 - Sling Load Operations - 3. (e) (4) |
| PB 005 | IFR training | OM D - (...) |
| PB 006 | Meteorological information | SOP 001 - Sling Load Operations - 3. (i) (2) 5. |
| PB 007 | Personal Protective Equipment (PPE) | SOP 001 - Sling Load Operations - 3. (k) |
| PB 008 | Radio contact with personnel on ground | SOP 001 - Sling Load Operations - 3. (d) (4) - Equipment - 3. (e) (2) (i) (L) - Training - 3. (e) (2) (ii) – Training - 3. (f) (1) (i) (G)/(J) – Training - 3. (i) (2) 7. – Operation - 3. (k) - Equipment |
| PB 009 | Initial Recce of the operating area | SOP 001 - Sling Load Operations - 3. (i) (1) |
| PB 010 | Previous ground reconnaissance | SOP 001 - Sling Load Operations - 3. (i) (1) |
| PB 011 | Pilot minimum mountain experience | SOP 001 - Sling Load Operations - 3. (e) (2) |
| PB 012 | Emergency procedures training | SOP 001 - Sling Load Operations - 3. (e) (3) (i) (A) - 3. (e) (3) (ii) (A) |
| PB 013 | Maintenance planning | CAMO |
| PB 014 | Personnel on ground experience | SOP 001 - Sling Load Operations - 3. (f) (1) (i) |
| PB 015 | Personnel on ground recency | SOP 001 - Sling Load Operations - 3. (f) (2) |
| PB 016 | Mission planning | SOP 001 - Sling Load Operations - 3. (i) (2) |
| PB 017 | Adverse weather policy/SOP | SOP 001 - Sling Load Operations - 3. (b) (1) - 3. (i) (2) 5. |
| PB 018 | Weather reports | SOP 001 - Sling Load Operations - 3. (i) (2) 5. |
| PB 019 | Pre-flight Risk Assessment | SOP 001 - Sling Load Operations - 3. (e) (2) (i) (Q) |
| PB 020 | In-flight decision making training | OM D CRM |
| PB 021 | Area qualification | OM D |
| PB 022 | Continuous Airworthiness Organisation (CAMO) | CAMO |
| PB 023 | IFR rated pilot | SOP 001 - Sling Load Operations - 3. (e) (1) (ii) |
| PB 024 | Safety Reports | Safety programme |
| PB 025 | EGPWS | Not implemented |
| PB 026 | Use of mirrors (sling load) | SOP 001 - Sling Load Operations - 3. (i) (1) |
| PB 027 | Use of suitable equipment | SOP 001 - Sling Load Operations - 3. (k) |
| PB 028 | Service and duty time | OM |
| RB 001 | Emergency Response Plan (ERP) | Safety programme |
| RB 002 | Pilot and personnel data | Operation Department |
| RB 003 | Safety Management System (SMS) | Safety programme |
| RB 004 | Safety reports | Safety programme |
| RB 005 | Spare helicopter availability | (...) |
| RB 006 | Personnel proper clothing | SOP 001 - Sling Load Operations - 3. (e) (2) (i) (K) |
| RB 007 | Survival kit | CAMO |
| RB 008 | Portable aeronautical radio on board | CAMO |
| RB 009 | Emergency Locator Beacon (ELT) | CAMO |
| RB 010 | Insurance | Helicopter documents |
| RB 011 | First Aid Kit | CAMO |
| RB 014 | Recurrent emergency procedures training | OM D |
| RB 015 | Unusual attitude training | OM D |
| RB 016 | IFR training | OM D |
| RB 017 | Inadvertent IMC training | OM D |
| RB 018 | Wire strike protection system | CAMO |