Courtesy: Elicampiglio

Photo: Michele Calovi

EHEST SAFETY MANAGEMENT TOOLKIT

HAZARDOUS EVENT IDENTIFICATION & RISK ASSESSMENT

HIRA - EXAMPLE – RADEC

1st Edition, Dcember 2015

HAZARDOUS EVENT IDENTIFICATION & RISK ASSESSMENT

HIRA 001

Sling Load Operations

(Change numbering and title accordingly)

- - E X A M P L E - -

Edition 1

December 2015

| Signature |  |  |  |  |
| --- | --- | --- | --- | --- |
| Position |  |  |  |  |
| Name |  |  |  |  |
|  | Issued by | Checked by | Approved by | Approved by |

# INDEX

1.	INDEX	3

2.	HIRA - RADEC	4

3.	LIST OF BARRIERS	7

# HIRA - RADEC

| RISK ASSESSMENT, DESCRIPTION, EVALUATION AND CONTROL (RADEC) |  |  |  |
| --- | --- | --- | --- |
| HIRA No.: PHA 001 | Definition: SLING LOAD OPERATIONS |  |  |
| Ref.: |  |  |  |
| Operation Description: Sling load operations in confined areas |  |  |  |
| Hazardous Events: Hazardous Event ID; Hazardous Event; Likelihood. HE 001; Reduced separation with obstacles; 2. HE 002; Reduced separation of the load from obstacles (sling load); 3. HE 003; Degraded aircraft performance; 1. HE 004; Degraded aircraft performance during sling load operations; 1. HE 005; Aircraft unintentionally deviates from normal in-flight parameters; 2. HE 006; Safety measures not respected by personnel on ground during sling load operations; 3. HE 007; Load is not properly prepared (sling load operations); 3. HE 008; Unexpected technical problem related to sling load operations; 3. HE 009; Unexpected technical problem on aircraft; 3. HE 010; Lose objects moved by the rotor downwash; 3. HE 011; Not accurate or missing mission planning and preparation (sling load); 2. HE 012; Flight close to, or into, adverse meteorological conditions; 2. HE 013; Unwanted movement of cargo in flight (sling load); 2. HE 014; Repetitive flight patterns/manoeuvres; 4. HE 015; External pressure; 3. |
| Consequences: Conseq. ID; Consequence; Risk. CO 001; Aircraft crash; 1 E. CO 002; Property damage on ground; 2 C. CO 003; Injury to personnel on ground; 2 C. CO 004; Injury to flight crew or crew members; 2 C. CO 005; Injury to passengers; 2 C. CO 006; Aircraft not available; 3 B. CO 007; Pilot/personnel not available; 3 B. CO 009; Rules infringement; 3 B. CO 010; Personnel grounded in an impervious area; 2 B. CO 011; Main rotor blades strike a ground obstacle; 2 C. CO 012; Tail rotor blades strike a ground obstacle; 2 C. CO 013; Impact with wires or cables; 1 E. CO 014; CFIT; 1 E. CO 015; Loss of tail rotor effectiveness; 1 B. CO 016; Loss of control with subsequent ground impact or aircraft damage; 1 D. CO 017; Payload damage during flight; 2 B. CO 018; Sling cable or load contact with obstacle on ground; 3 B. CO 019; Sling cable or load get caught with obstacle on ground; 2 C. CO 021; Engine failure – Single-engine; 1 D. CO 022; Emergency cargo release; 2 B. CO 023; Unintentional cargo release; 2 B. CO 024; Object detached lose from cargo; 2 B. CO 025; FOD; 2 B. CO 026; Unplanned landing; 2 B. CO 028; Unexpected reduced performance; 2 C. CO 029; Load swings in flight; 3 B. CO 030; Sling cable or load contact with the aircraft structure during flight; 1 D. |
| Barriers in place: Proactive Barrier ID; Proactive Barrier. PB 001; SOP Sling Load Operations. PB 002; Sling load operation training. PB 003; Pilot experience. PB 004; Pilot recency. PB 005; IFR training. PB 006; Meteorological information. PB 007; Personal Protective Equipment (PPE). PB 008; Radio contact with personnel on ground. PB 009; Initial Recce of the operating area. PB 010; Previous ground reconnaissance. PB 011; Pilot minimum mountain experience. PB 012; Emergency procedures training. PB 013; Maintenance planning. PB 014; Personnel on ground experience. PB 015; Personnel on ground recency. PB 016; Mission planning. PB 017; Adverse weather policy/SOP. PB 018; Weather reports. PB 019; Pre-flight Risk Assessment. PB 020; In-flight decision making training. PB 021; Area qualification. PB 022; Continuous Airworthiness Organisation (CAMO). PB 023; IFR rated pilot. PB 024; Safety Reports. PB 025; EGPWS. PB 026; Use of mirrors (sling load). PB 027; Use of suitable equipment. PB 028; Service and duty time. Recovery Barrier ID; Recovery Barrier. RB 001; Emergency Response Plan (ERP). RB 002; Pilot and personnel data. RB 003; Safety Management System (SMS). RB 004; Safety reports. RB 005; Spare helicopter availability. RB 006; Personnel proper clothing. RB 007; Survival kit. RB 008; Portable aeronautical radio on board. RB 009; Emergency Locator Beacon (ELT). RB 010; Insurance. RB 011; First Aid Kit. RB 014; Recurrent emergency procedures training. RB 015; Unusual attitude training. RB 016; IFR training. RB 017; Inadvertent IMC training. RB 018; Wire strike protection system. |
| Additional Barriers |  |  |  |
| Additional Barriers: |  |  | Implemented? |
| Is the residual Safety Risk Acceptable? (see Safety Risk Matrix) |  |  |  |
| ACCEPTABLE | TOLERABLE | UNACCEPTABLE |  |
| Is the residual risk acceptable: YES NO (if NO go back to previous section) RISK ASSESSMENT CLOSED |  |  |  |

# LIST OF BARRIERS

OPERATION No.: OP 01	OPERATION DESCRIPTION: Sling load operations

| BAR. REF. | DESCRIPTION | REF. IN SOP/DOCUMENTS |
| --- | --- | --- |
| PB 001 | SOP Sling Load Operations | SOP 001 - Sling Load Operations |
| PB 002 | Sling load operation training | SOP 001 - Sling Load Operations - 3. (e) (2) - 3. (e) (4) - 3. (f) (1) - 3. (f) (2) OM D - (...) |
| PB 003 | Pilot experience | SOP 001 - Sling Load Operations - 3. (e) (3) |
| PB 004 | Pilot recency | SOP 001 - Sling Load Operations - 3. (e) (4) |
| PB 005 | IFR training | OM D - (...) |
| PB 006 | Meteorological information | SOP 001 - Sling Load Operations - 3. (i) (2) 5. |
| PB 007 | Personal Protective Equipment (PPE) | SOP 001 - Sling Load Operations - 3. (k) |
| PB 008 | Radio contact with personnel on ground | SOP 001 - Sling Load Operations - 3. (d) (4) - Equipment - 3. (e) (2) (i) (L) - Training - 3. (e) (2) (ii) – Training - 3. (f) (1) (i) (G)/(J) – Training - 3. (i) (2) 7. – Operation - 3. (k) - Equipment |
| PB 009 | Initial Recce of the operating area | SOP 001 - Sling Load Operations - 3. (i) (1) |
| PB 010 | Previous ground reconnaissance | SOP 001 - Sling Load Operations - 3. (i) (1) |
| PB 011 | Pilot minimum mountain experience | SOP 001 - Sling Load Operations - 3. (e) (2) |
| PB 012 | Emergency procedures training | SOP 001 - Sling Load Operations - 3. (e) (3) (i) (A) - 3. (e) (3) (ii) (A) |
| PB 013 | Maintenance planning | CAMO |
| PB 014 | Personnel on ground experience | SOP 001 - Sling Load Operations - 3. (f) (1) (i) |
| PB 015 | Personnel on ground recency | SOP 001 - Sling Load Operations - 3. (f) (2) |
| PB 016 | Mission planning | SOP 001 - Sling Load Operations - 3. (i) (2) |
| PB 017 | Adverse weather policy/SOP | SOP 001 - Sling Load Operations - 3. (b) (1) - 3. (i) (2) 5. |
| PB 018 | Weather reports | SOP 001 - Sling Load Operations - 3. (i) (2) 5. |
| PB 019 | Pre-flight Risk Assessment | SOP 001 - Sling Load Operations - 3. (e) (2) (i) (Q) |
| PB 020 | In-flight decision making training | OM D CRM |
| PB 021 | Area qualification | OM D |
| PB 022 | Continuous Airworthiness Organisation (CAMO) | CAMO |
| PB 023 | IFR rated pilot | SOP 001 - Sling Load Operations - 3. (e) (1) (ii) |
| PB 024 | Safety Reports | Safety programme |
| PB 025 | EGPWS | Not implemented |
| PB 026 | Use of mirrors (sling load) | SOP 001 - Sling Load Operations - 3. (i) (1) |
| PB 027 | Use of suitable equipment | SOP 001 - Sling Load Operations - 3. (k) |
| PB 028 | Service and duty time | OM |
| RB 001 | Emergency Response Plan (ERP) | Safety programme |
| RB 002 | Pilot and personnel data | Operation Department |
| RB 003 | Safety Management System (SMS) | Safety programme |
| RB 004 | Safety reports | Safety programme |
| RB 005 | Spare helicopter availability | (...) |
| RB 006 | Personnel proper clothing | SOP 001 - Sling Load Operations - 3. (e) (2) (i) (K) |
| RB 007 | Survival kit | CAMO |
| RB 008 | Portable aeronautical radio on board | CAMO |
| RB 009 | Emergency Locator Beacon (ELT) | CAMO |
| RB 010 | Insurance | Helicopter documents |
| RB 011 | First Aid Kit | CAMO |
| RB 014 | Recurrent emergency procedures training | OM D |
| RB 015 | Unusual attitude training | OM D |
| RB 016 | IFR training | OM D |
| RB 017 | Inadvertent IMC training | OM D |
| RB 018 | Wire strike protection system | CAMO |